WEEK 6

Learn Online

Metal Welding

- Oxyacetylene Welding

  - Uses a flame from an oxygen-acetylene gas mixture.

  - A filler rod is melted into a weld pool on the parent metal.

  - Common for manual, portable welding tasks.

- Metal Inert Gas (MIG) Welding

  - Uses a consumable wire electrode with a gas shield (usually Argon).

  - An electric arc melts the electrode to form the weld.

  - Suitable for automated and high-speed welding.

- Electron Beam Welding

  - A focused electron beam melts the metal in a vacuum.

  - Deep and precise welds, with minimal distortion.

  - Requires close fit-up; used in aerospace/nuclear sectors.

🧱 Ceramic Welding

- Diffusion Bonding

  - Applies high pressure and temperature in a furnace.

  - Materials join without melting; atoms diffuse across the bond line.

  - Ideal for ceramics and dissimilar material joints.

🧵 Polymer Welding

- Induction Welding

  - Uses an induction coil to generate heat in a metal insert.

  - Thermoplastics are fused around the heated insert.

  - Common for joining tubes in plastic systems.

- Laser Welding (Polymer)

  - A laser beam focuses on a polymer sheet with a light-absorbing dye.

  - Heat melts and fuses the layers at the weld line.

  - Enables clean, precise polymer joints.

No External Bolts or Holes

- In traditional assembly, you'd see mechanical fasteners (bolts, rivets) that pierce through flanges.

- This method weakens the structure and disrupts the smooth outer surface (bad for aesthetics & aerodynamics).

Flanged Polymer Parts with Adhesive Bonding

- Instead of bolting, the yacht's panels are designed with flanges that fit together like puzzle pieces.

- A solvent-based adhesive is sprayed using a compressed air spray gun.

- The adhesive is applied within the flange seam — all around the part.

- Pressure is applied to hold parts together during curing.

Advantages:

- No visible joins or holes → sleek, waterproof surface.

- Stronger joints → avoids stress concentration from bolt holes.

- Lightweight and corrosion-resistant → ideal for marine applications.

- Cleaner aesthetic → luxury finish.

TYPES OF ADHESIVE

Types of Adhesive - Explained

1. Cyanoacrylate (Super Glue)

- Very fast-setting, bonds in seconds.

- Great for small parts, plastics, rubber, and skin.

- Brittle under shock or high heat.

- Cures with moisture in the air.

2. Epoxy

- Two-part adhesive: resin + hardener.

- Very strong, durable, resistant to chemicals and heat.

- Excellent for metal, composites, and structural bonding.

- Slow to cure (minutes to hours).

3. Polyurethane

- Flexible and durable.

- Bonds a wide range of materials: metal, wood, plastics, glass.

- Moisture-cured and often used in construction and automotive.

4. Silicone

- Flexible, rubbery adhesive.

- Excellent for sealing joints in moist or high-temperature environments.

- Common in electronics, kitchens, and windows.

5. Anaerobic Adhesives

- Cure in the absence of air (oxygen).

- Used to seal threads, gaskets, and bearings (e.g., Loctite).

- Great for metal-metal joins.

6. Toughened Acrylic

- Modified acrylics for improved toughness.

- Excellent impact resistance and strong bonding.

- Used in aerospace, automotive, and sports equipment.

7. Phenolic

- Thermosetting resin, often used as a binder in wood products (e.g., plywood).

- High-temperature resistance, but brittle.

- Used in aerospace composites and laminates.

8. MS Polymer (Modified Silane Polymer)

- Hybrid adhesive combining silicone and polyurethane properties.

- UV and weather resistant.

- Used in construction and transport (e.g., trains, yachts).

9. Polysulphide

- Flexible, resistant to fuels and chemicals.

- Ideal for aerospace fuel tanks, marine sealing.

- Cures slowly.

10. Hot Melt

- Thermoplastic adhesive applied hot (melted), then solidifies on cooling.

- Quick bonding, used in packaging, crafts, and furniture.

- Low strength but fast and clean.

11. PVA (Polyvinyl Acetate)

- White glue / wood glue.

- Common in woodworking, paper, and crafts.

- Water-based and easy to use, but weak in wet or hot conditions.

12. UV Cure

- Cures (hardens) when exposed to ultraviolet light.

- Used in electronics, glass, and medical devices.

- Fast and precise with clear adhesives.

| Adhesive | Key Use Area | Strength | Cure Time | Special Trait |
| --- | --- | --- | --- | --- |
| Cyanoacrylate | Quick fixes, small parts | High | Seconds | Moisture-cured |
| Epoxy | Structural bonding | Very High | Slow (hrs) | Heat/chemical resistant |
| Polyurethane | Versatile construction | Medium | Medium | Flexible, waterproof |
| Silicone | Sealing in moist environments | Low | Slow (24 hrs) | Flexible, high-temp |
| Anaerobic | Thread locking in metals | Medium | Slow | Cures without air |
| Toughened Acrylic | Aerospace, impact resistance | High | Medium | Shock-resistant |
| Phenolic | Composites, wood panels | High | Heat required | Brittle, heatproof |
| MS Polymer | Building & transport | Medium | Medium | UV/weather resistant |
| Polysulphide | Aerospace, marine | Medium | Slow | Fuel/chemical resistant |
| Hot Melt | Packaging | Low | Seconds | Fast, low strength |
| PVA | Wood, crafts | Low | Slow | Water-soluble |
| UV Cure | Electronics, medical | Medium | Seconds (w/UV) | Needs UV light to cure |

1. Polyurethane

- Requirements: Moisture to cure (some are two-part).

- Strengths: Flexible, impact resistant, good for bonding different materials.

- Uses: Automotive, construction, footwear.

- Surface prep: Clean, may require primer for some plastics or metals.

2. Epoxy

- Requirements: Two-part mix (resin + hardener).

- Strengths: Very strong, chemical and heat resistant.

- Uses: Structural bonding in aerospace, electronics, marine.

- Surface prep: Clean, roughened surface improves bond.

3. MS Polymer (Modified Silane)

- Requirements: Cures with moisture.

- Strengths: UV resistant, flexible, paintable.

- Uses: Construction, sealing, bonding of panels.

- Surface prep: Clean, dry, some primers may be used.

4. PVA (Polyvinyl Acetate)

- Requirements: Water-based, cures as water evaporates.

- Strengths: Easy to use, non-toxic.

- Uses: Woodworking, paper, fabrics.

- Surface prep: Clean, dry, porous surfaces preferred.

5. Polysulphide

- Requirements: Often two-part, cures chemically.

- Strengths: Flexible, fuel-resistant.

- Uses: Aerospace, glazing, marine sealants.

- Surface prep: Clean, primer may be required for some substrates.

6. Cyanoacrylate (Super Glue)

- Requirements: Moisture in air to cure.

- Strengths: Rapid bonding, strong in tension.

- Uses: Plastics, rubbers, electronics, medical devices.

- Surface prep: Dry and clean; acidic surfaces can slow cure.

7. UV Cure

- Requirements: UV light exposure to cure.

- Strengths: Fast cure, precision bonding.

- Uses: Medical, electronics, glass.

- Surface prep: Transparent or UV-transmitting surfaces.

8. Anaerobic

- Requirements: Absence of oxygen and presence of metal ions.

- Strengths: Cures between metal parts (threadlockers, flange sealants).

- Uses: Mechanical assemblies (bolts, shafts).

- Surface prep: Clean metal surfaces, no large gaps.

9. Toughened Acrylic

- Requirements: Two-part, or primer-activated systems.

- Strengths: Tough, fast, can bond oily surfaces.

- Uses: Automotive, aerospace, structural bonding.

- Surface prep: Minimal, tolerates surface contaminants.

10. Silicone

- Requirements: Cures with moisture or heat.

- Strengths: Flexible, high-temperature and chemical resistant.

- Uses: Electronics, automotive, cookware, seals.

- Surface prep: Clean surfaces, sometimes a primer is used.

11. Hot Melt

- Requirements: Heat applied to melt adhesive, cools to solidify.

- Strengths: Fast setting, easy application.

- Uses: Packaging, furniture, textiles.

- Surface prep: Clean and dry; mostly for porous or non-critical surfaces.

12. Phenolic

- Requirements: Heat and pressure to cure (thermosetting).

- Strengths: Very high temperature and chemical resistance.

- Uses: Aerospace, electronics, industrial composites.

- Surface prep: Clean, often requires pressure bonding during cure

What is Surface Wetting?

Surface wetting describes how well a liquid (like glue or paint) spreads out on a surface. Good wetting = strong bond.

- Higher surface energy → better wetting → stronger adhesive bonding.

- Low surface energy (e.g. PTFE/Teflon) resists wetting — glue rolls

Types of Loads & Stress Distribution

Left Side: Types of Loads on Joints

These show different ways adhesive joints can be stressed:

- Tensile Load (pulling apart directly) – good distribution

- Shear Load (sliding past each other) – good

- Compressive Load – also well distributed

- Peel Load (one side lifting away) – very poor

- Cleavage Load (prying open) – very poor

✅ Good loads (tensile, shear, compressive) spread stress evenly.
❌ Bad loads (peel, cleavage) create stress peaks at the ends, which can cause early failure.

Right Side: Stress Distribution Graphs

These charts show how stress is spread across the bond area:

- Ideal: flat line – stress evenly distributed.

- Peel/Cleavage: stress is concentrated at the edge → leads to crack initiation.

🔍 Slide 2: Design to Reduce Stress Peaks

This slide shows bad vs. improved joint designs when peel or cleavage loads are unavoidable.

Left Column: Peel Load Problems

- Joints stressed in peel (e.g., top flaps being peeled open).

- Leads to concentrated stress at the edge.

Right Column: Better Design Solutions

- Add flanges, fillets, or overlaps to redirect the load from peel → to shear or compression.

- Wrap the adhesive around the load path so it's not peeling but being sheared.

- Use continuous bonding areas rather than single-point joins.

✅ Key Takeaways

- Avoid peel and cleavage stresses in design.

- Distribute loads across large areas, preferably in shear or compression.

- Use smart geometric design (like overlaps or wraps) to redirect forces.

- Minimising stress peaks extends the life and safety of adhesive joints.

ROD AND TUBE JOINT DESIGN

Straight Butt Joint

🧩 Use: Good for rods in tension and compression.
⚠️ Weakness: Poor in bending unless the bond area is large.
✅ Simple but not good for flexing loads.

2. Angled Insertion Joint

🧩 Use: Good for rods only (not tubes).
✅ Angled insertion gives some mechanical interlock.
⚠️ Requires precise shaping.

3. Tube-in-Tube (Socket) Joint

🧩 Use: Good for rods, where one part fits inside another.
✅ Easy to align, has moderate bonding area.
⚠️ May not suit all diameters.

4. Sleeved Lap Joint

🧩 Use: Works well for rods or tubes.
✅ Balanced and stronger due to large contact area.
⚠️ Slightly more complex to fabricate.

5. Tapered Insertion Joint

🧩 Use: Generally poor for tubes.
⚠️ Tapered ends can focus stress and reduce strength.
❌ Avoid in critical structural joints.

6. Pinned Collar Joint

🧩 Use: Good for rods but slower to produce.
✅ Adds mechanical fixing (pin) for added strength.
⚠️ More time-consuming, better for high-strength needs.

7. Threaded End Joint

🧩 Use: Good for tubes if wall thickness allows machining.
✅ Secure and removable connection.
⚠️ Requires precision cutting and thicker walls.

8. Elbow Joint

🧩 Use: For corners in rods or tubes.
✅ Elbows offer neat 90° redirection.
⚠️ Best used with prefabricated elbow fittings for strength and ease.

Cylindrical Bonding

🔧 What is it?

- A method of joining cylindrical parts (e.g. gears, shafts) using press-fit + adhesive.

- Commonly used in Scania truck gearboxes, where gear sets are press-bonded onto a countershaft.

💡 How it works:

- One part is slightly larger and pressed into another (interference fit).

- An anaerobic adhesive (like Loctite) fills microscopic gaps.

- It cures in the absence of oxygen, forming a solid bond.

✅ Advantages:

- Even load distribution across the surface.

- Prevents fretting (microscopic movement leading to wear).

- High torque resistance without machining keyways or splines.

Threadsealing

🔧 What is it?

- Involves applying sealant to threaded parts (nuts, bolts, pipe threads).

- Prevents leakage of gases or fluids under pressure.

🛢️ Example shown:

- Petrol pump thread sealing — a safety-critical application.

- Uses anaerobic adhesives designed to:

  - Fill gaps between threads,

  - Prevent leaks,

  - Lock threads in place to resist vibration loosening.

✅ Advantages:

- Leak-proof and vibration-proof.

- Chemical and pressure resistant.

- No need for tapes or mechanical seals.

# Additive Manufacturing Techniques Summary

| Technique | How It Works | Key Feature | Typical Materials | Example Applications |
| --- | --- | --- | --- | --- |
| Selective Laser Sintering (SLS) | A laser selectively sinters powdered polymer, layer by layer. | Laser sinters powder layer-by-layer (polymers) | Nylon, ABS, polycarbonate | Power tool housings, circuit breakers, aerospace handles |
| Selective Laser Sintering of Metals | A laser partially melts metal powder layer-by-layer. The porous part is then infiltrated with bronze. | Laser sinters metal powder, then infiltrated with bronze | Stainless steel, titanium + bronze | Injection mould tools with cooling channels, die casting moulds |
| Stereo-lithography (SLA) | A UV laser cures layers of liquid photopolymer resin. | Uses UV light to cure liquid photopolymer | Photopolymers (acrylates) | Phone cases, implants, EDM electrodes |
| Laminated Object Manufacturing (LOM) | Layers of material are bonded and laser-cut into shape. | Bond and laser-cut sheets layer by layer | Paper, polymer, metal sheets | Casting moulds, automotive mock-ups |
| Solid Ground Curing (SGC) | UV light cures resin through masks to form each layer simultaneously. | UV-curing resin through masks, layer by layer | Photosensitive resin, wax | Detailed prototypes, electronics enclosures |
| Fused Deposition Modelling (FDM) | A heated nozzle extrudes molten thermoplastic filament. | Heated nozzle extrudes thermoplastic filament | ABS, Nylon, PLA | Rapid part prototypes, functional models |
| Ballistic Particle Manufacturing (BPM) | Molten droplets are jetted by piezo system and solidified layer by layer. | Molten droplets jetted by piezo system and frozen | Thermoplastics | Experimental rapid prototyping |
| 3D Printing (Inkjet-based) | Inkjet heads deposit tiny polymer droplets layer by layer. | Multiple jets deposit polymer like an inkjet printer | Thermoplastic polymer | Basic concept models and fast prints |

| Process | Key Method | Materials Used | How to Distinguish It | Example Use |
| --- | --- | --- | --- | --- |
| SLS (Polymers) | Laser sintering of polymer powders layer by layer | ABS, Nylon, Polycarbonate | Uses powder + laser, no support material needed. Fast and accurate. | Airframe parts, housings, spotlight parts |
| SLS (Metals) | Laser sintering of metal powders + infiltrated with bronze | Stainless steel, Titanium | Same as polymer SLS but for metals. Post-infiltration with bronze. | Injection mould tools with cooling channels |
| SLA (Stereolithography) | UV laser cures liquid photopolymer layer by layer | Acrylates (photopolymers) | Liquid resin, not powder. Requires post-curing. Very smooth finish. | Bone implants, phone cases, EDM electrodes |
| LOM (Laminated Object Manufacturing) | Laser cuts and bonds sheet layers (paper/polymer/metal) | Paper, polymer sheets, thin metal sheets | Feeds from a roll, then laser cuts each layer. Looks like lamination. | Visual models, casting patterns |
| SGC (Solid Ground Curing) | UV light cures resin via a photocopier-like mask | Photosensitive resin | Entire layer cured at once. Wax supports applied and chilled. | Detailed electronic enclosures |
| FDM (Fused Deposition Modelling) | Heated nozzle extrudes filament like a glue gun | ABS, Nylon, PLA | Most common home 3D printer. Heated nozzle + filament roll. | Functional prototypes, low-cost models |
| BPM (Ballistic Particle Mfg.) | Piezo-jet shoots molten microparticles that freeze | Thermoplastics | Think of tiny droplets sprayed and frozen. White finish only. | Experimental prototyping |
| 3D Printing (Inkjet-based) | Multiple inkjets deposit polymer droplets | Thermoplastics | Uses inkjet heads like a printer. Very fast. | Concept models, early visual prototypes |