# Ceramics – Multiple Choice Questions with Answers

1. Which of the following is NOT a typical feature of ceramics?

a) High hardness

b) Good thermal insulation

c) High ductility

d) Low electrical conductivity

Answer: c) High ductility

2. Which type of bonding is most commonly found in ceramics?

a) Metallic

b) Ionic and covalent

c) Hydrogen

d) Van der Waals

Answer: b) Ionic and covalent

3. What is the primary raw material used in traditional ceramics?

a) Silica

b) Alumina

c) Clay

d) Magnesia

Answer: c) Clay

4. What is the term for the process of heating compacted ceramic powders to cause bonding?

a) Sintering

b) Firing

c) Baking

d) Annealing

Answer: a) Sintering

5. Which property is generally low in ceramics?

a) Compressive strength

b) Hardness

c) Electrical conductivity

d) Thermal stability

Answer: c) Electrical conductivity

6. Which ceramic is widely used for electrical insulation?

a) SiC

b) Al₂O₃

c) WC

d) ZrO₂

Answer: b) Al₂O₃

7. Why are ceramics generally brittle?

a) Weak bonding

b) Grain boundary slip

c) Lack of dislocation movement

d) High porosity

Answer: c) Lack of dislocation movement

8. Which of these ceramics is most commonly used in cutting tools?

a) ZrO₂

b) SiC

c) WC

d) TiO₂

Answer: c) WC

9. What is the purpose of using binders in ceramic processing?

a) Reduce grain size

b) Increase electrical conductivity

c) Improve green strength

d) Enhance porosity

Answer: c) Improve green strength

10. Which property makes ceramics useful in thermal insulation?

a) High density

b) Low thermal conductivity

c) High reflectivity

d) High ductility

Answer: b) Low thermal conductivity

11. Which of the following is used in bio-ceramic applications?

a) WC

b) Si₃N₄

c) Hydroxyapatite

d) SiO₂

Answer: c) Hydroxyapatite

12. Which material is known for both high hardness and high thermal conductivity?

a) SiO₂

b) Diamond

c) Al₂O₃

d) Graphite

Answer: b) Diamond

13. Which of the following ceramics is piezoelectric?

a) ZrO₂

b) PZT

c) WC

d) MgO

Answer: b) PZT

14. What is meant by 'green body' in ceramics?

a) Sintered body

b) Unfired compact

c) Fully dense body

d) Crack-free ceramic

Answer: b) Unfired compact

15. Which method is used to achieve finer ceramic particle sizes?

a) Annealing

b) Sintering

c) Ball milling

d) Precipitation

Answer: c) Ball milling

16. Which property improves with better particle packing?

a) Porosity

b) Density

c) Brittleness

d) Electrical resistance

Answer: b) Density

17. Which phase sintering involves neck growth without shrinkage?

a) Liquid

b) Solid

c) Vapour

d) Reactive liquid

Answer: c) Vapour

18. Which ceramic material is used for high-speed bearings?

a) Si₃N₄

b) WC

c) PZT

d) AlN

Answer: a) Si₃N₄

19. Which property of ceramics is essential for medical implants?

a) Electrical conductivity

b) Biocompatibility

c) Porosity

d) Brittleness

Answer: b) Biocompatibility

20. Which of the following is NOT a typical application of technical ceramics?

a) Electrical insulators

b) Gears

c) Bearings

d) Conducting wires

Answer: d) Conducting wires