# Advantages and Disadvantages of Polymer Shaping Methods

| Method | Advantages | Disadvantages |
| --- | --- | --- |
| Injection Moulding | High precision and repeatability Fast cycle times for mass production Good surface finish | High tooling cost Limited to thermoplastics Difficult for very large parts |
| Gas-Assisted Injection Moulding | Hollow parts with less material Reduces warping and sink marks Good for complex shapes | More complex tooling Process control is critical Only for thermoplastics |
| Extrusion | Continuous process Excellent for long, uniform profiles Low material waste | Limited to simple cross-sections Surface finish may need post-processing |
| Blown Film Extrusion | Produces thin, biaxially oriented films High throughput Ideal for packaging | Limited to thin film products Requires precise control |
| Blow Moulding | Great for hollow containers (e.g., bottles) Low cost for large volumes Good for thin-walled parts | Poor control over wall thickness Limited to simple shapes |
| Vacuum Forming | Low tooling costs Fast setup for low-volume production Simple equipment | Limited to shallow parts Poor detail resolution Only one good surface |
| Rotational Moulding | Uniform wall thickness Ideal for large hollow items No seams or welds | Long cycle time Lower detail accuracy Not suitable for small intricate parts |
| Compression Moulding | Good for thermosets Low scrap rate Can mould large parts | Limited to simple geometries Slower than injection moulding Labour intensive |